****************************************************

Get Module Information

****************************************************

UI:	$C00#

- No Information

MD:$R00#

- When Push Button On Relay In Relay Learning Mode

MD:	$R01aabbcc#

aa=ID1

bb=ID2

cc=ID3

- When Push Button On Relay In Sensor Learning Mode

MD:	$R04a#

a:0	Truck Head

a:1	Truck Tail 1

a:2	Truck Tail 2

- When Sensor Signal Input In Sensor Learning Mode

MD:	$R04aa#

aa=Tire Position  (0x00-0x23)

****************************************************

Set Relay Learning

****************************************************

UI:	$C01x#

MD:	$R01x#

x:0	Truck Head

x:1	Truck Tail 1

x:2	Truck Tail 2

When Push Button On Relay In Relay Learning Mode

MD:	$R01aabbcc#

aa=ID1

bb=ID2

cc=ID3

****************************************************

Set Sensor Learning

****************************************************

UI:	$C04#

MD:	$R04#

- When Push Button On Relay In Sensor Learning Mode

MD:	$R04x#

x:0	Truck Head

x:1	Truck Tail 1

x:2	Truck Tail 2

- When Sensor Signal Input In Sensor Learning Mode

MD:	$R04aa#

aa=Tire Position  (0x00-0x23)

****************************************************

Exit Learning Mode

****************************************************

- UI:	$C02#

MD:	$R02#

- MD:	$R02#

UI:	None

****************************************************

Set Truck Type

****************************************************

UI:	$C03abbccddeff#

a:	Truck Type

a:0 	Truck without relay

a:1 	Truck Without Tail

a:2 	Truck With 1 Tail

a:4 	Truck With 2 Tail

bb:	Truck Segment 1 Tire Type

cc:	Truck Segment 2 Tire Type

dd:	Truck Segment 3 Tire Type

e:	Auto Link Flag

e:0 No Auto Link

e:1 Auto Link

ff:	Check Sum

MD:	$R03#

#Check Sum=‘C’+’0’ +’3’ +’a’ +’b’ +’b’ +’c’ +’c’ +’d’ +’d’ +’e’

****************************************************

Get Tire Data

****************************************************

UI:	$C05AA#

MD:	$R05aabbccddee#

aa:	Tire Position  (0x00-0x23)

bb:	Tire Status Flag

bit0:		Half Psi

bit1:		Battery Low Indication

bit2:		Data Flashed Indication from Sensor

bit3:		Data Flashed Indication from Relay

bit4:		Low Pressure Indication

bit5:		High Pressure Indication

bit6:		High Temperature Indication

bit7:		No Use

cc:	Tire Temperature	(0x00->0xA5,	-40 To 125 Degree C)

dd:	Tire Pressure		(0x00->0xFA	0 To 250 PSI)

ee:	Check sum		(0x00->0xFF)

****************************************************

Get Relay Status

****************************************************

UI:	$C06#

MD:	$R06aabb#

aa:	Relay Status Flag

bit0:		Relay 1 Connected Indication

bit1:		Relay 2 Connected Indication

bit2:		Relay 3 Connected Indication

bit3:		Relay 1 Battery Low Indication

bit4:		Relay 2 Battery Low Indication

bit5:		Relay 3 Battery Low Indication

bb:	Check sum			(0x00->0xFF)

****************************************************

Set Tire Threshold

****************************************************

UI:	$C08aabbccddee#

aa:	Tire Position  (0x00-0x23)

bb:	Pressure Low Threshold (0x00->0xB4,0 to 180 PSI)

cc:	Pressure High Threshold (0x00->0xB4,0 to 180 PSI)

dd:	Temperature High Threshold (0x00->0x7D,	0 to 125 Degree C)

ee:	Check sum			(0x00->0xFF)

MD:$R08#

;--------------------------